Closed Door Baler with Hinged Sidewall

Model CD-738-HS-20

Revised: 3/09

# HYDRAULIC

Main Pumps	42 GPM Total Flow

System Pressure	3000 PSI

Main Cylinder	7 In Bore, 106 In Stroke, 4 In Rod

Completely Ejects Finished Bale

Door Cylinder	4 In Bore

Hinge Side Cylinder	4 In Bore

Total Force	115,400 Lb

Ram Face Pressure	93 PSI

Oil Reservoir	100 Gallon Capacity

Filtration	10 Micron Return Line  Filter

## ELECTRICAL

Main Motor	20 HP, 460 Volt, 3 Phase (Standard)

Controls  	All Control Circuits 120 Volts

Photocell	Infrared Type

Motor Type	TEFC, High Efficiency, Across The Line Starting is

Standard

# STRUCTURAL

Length Overall	242 In

Width Overall	95 In

Overall Height	152 In

Charge Box Opening	54 In Wide x 35 In Long

Shear Blade	T-1 Alloy Steel

Shipping Weight	12,000 Lb Approximate

Floor of Baler is Long Wearing T-1 Plate, Bottom of Platen is T-1 Plate

# PERFORMANCE

Bale Size	30 In Wide x 43 In High x 60 In Length

Bale Volume 	44.8 Cubic Feet Expanded (approximately)

Cycle Time	49 Seconds (No Load - Full Stroke)

Five Wire-Tie Slots

| Material | Bale Weight | Density – PCF | Bales per Hour |
| --- | --- | --- | --- |
| O.C.C. | up to 1350 LB | Up To 30 | up to 5 |
| News | up to 1400 LB | Up To 31 | up to 6 |
| CPO & High Grades | up to 1430 LB | Up To 32 | up to 6 |
| Bulk Office Waste | up to 1400 LB | Up To 31 | up to 6 |
| PET (Plastic) | up to 1000 LB | Up To 22 | up to 3 |
| HDPE (Plastic) | up to 1050 LB | Up To 23 | up to 3 |
| UBC (Aluminum Cans) | up to 600 LB | Up To 13 | up to 4 |
| Steel Cans | up to 1075 LB | Up To 24 | up to 6 |

*Performance Rates, Bale Weights, and Bale Densities Are Subject to Moisture Content, Material Pre-Bale Densities, Feed Rates, And Other Variables in Baling

Standard Features			Available Options

Programmable Controller			Oil Cooler For 30 HP or Less

Power Pack Mounted on Baler			Conveyor Controls

Bale  Runout Table (6 Ft Long)			Oil Heater

Voltage: 460/3/60 Across Line			Contact Factory for Other Available Options

Voltage Upcharge For Non-Standard Voltage

## Specifications and Design are Subject to Change if, in the opinion of the Manufacturer, the Change is an improvement.